|  | Modeling and Automation of Industrial Processes MEEC/MEGE - 2021 / 2022 |  | Group: ____ __-_____ __-_____ __-_____ __-_____ |
| --- | --- | --- | --- |

2nd Lab. - Alarm System as a Discrete Event System

Part B – Stochastic Simulation and Markov Chains

This laboratory assignment aims at studying Discrete Event Systems (DESs) in the aspects of modeling and analysis of properties. This assignment develops the previous assignment by describing its high-level system as a DES and studying its operation.

The main objective of this last phase, part B, is the application of stochastic simulation to assess system operation, namely permanence (percentage of time) the system is in each state. Alternatively, the Petri net system with stochastic transitions can also be described as a Markov Chain (MC), from which the permanence may also be estimated.

In this part of the work, the Petri net C=(P,T,D+,D-,μ0) describing the intrusion alarm system is simplified to have a small incidence matrix. The number of places (conditions) is reduced to three:

p1  –  OFF mode

p2  – Presence detection mode

p3  – Alarm mode

The set of transitions is T={t1,  t2,  t3,   t4,  t5,  t6,  t7} and the incidence matrix is:

D=(-1+1)-1-10+10+10+10(-1+1)-10000+100(-1+1)-1

where D=D+-D-, and an entry Dij=-Dij-+Dij+=(-1+1) denotes two arcs, namely (pi,tj) and (tj,pi). the initial marking is μ0=1  0  0T.

The tools mostly used in this work, part B, run in MATLAB / JAVA. Using the login information indicated in the course webpage, see in the course SVN:

- The graphical freeware editor "PIPE2" which allows creating Petri Net models to import with the MATLAB toolbox "TPN5", function rdp (use the version available in the SVN)

- MATLAB functions simulating Petri nets, in particular the "5 Philosophers".

B1. [Graphical representation] Draw the Petri net proposed in this guide, part B, using the PIPE2 editor. Simulate by hand a small sequence of events, show the reached states, therefore demonstrating the functionality of the net.

B2. [Reachable Set] Indicate the reachable set, R(C,μ0), of the Petri net. While in general a Petri net cannot be represented as a Finite State Machine (FSM), can the Petri net C proposed in this part of the work be represented as an FSM?

B3. [Stochastic firing vector] Let P(ti) denote the probability of firing transition ti. Let the following characterize all transitions:

Pt1=α1=0.06

Pt2=α2=0.45

Pt4=β=0.92

Pt6=γ=0.84

which imply Pt3=1-(α1+α2),  Pt5=1-β and Pt7=1-γ. This probabilistic definition allows running a Petri net in a stochastic way, not having to manually define the firing of transitions, by setting the firing vector with the following code:

alpha1=0.06; alpha2=0.45; beta=0.92; gamma=0.84;

a= rand(1); b= rand(1); c= rand(1);

q1= (a<alpha1);

q2= (alpha1<=a & a<alpha1+alpha2);

q3= (alpha1+alpha2 <= a);

qk= [q1 q2 q3 (b<beta) (beta<=b) (c<gamma) (gamma<=c)]';

Use this firing vector in the simulation of the Petri net defined in this part, part B, of the assignment (see also suggestions in Annex A2). After the simulation, compute the percentage of the total time the Petri net stays in each state μ∈R(C,μ0).

B4. [Markov chain, right stochastic matrix] Alternatively to simulating the Petri net with stochastic instances of the firing vector, one may compute the percentage of time the net stays in a state using a Markov chain formalism . Considering the states of a Petri net, μ1,μ2,μ3∈R(C,μ0), one forms a Markov chain state vector

xkT=P(μ1)  P(μ2)  P(μ3)T

where P(μi) denotes the probability of the Petri net being at state μi∈R(C,μ0). The dynamic equation of the Markov chain is xk+1T=xkTA. In the Petri net here considered, we have

A=α1α21-(α1+α2)1-ββ01-γ0γ .

Matrix A is known as a right stochastic matrix, meaning that each line sums to one. In case the Petri net starts at state μ1, one has x0T=1 0 0T. As k grows to infinity, the state xkT converges to a steady value simply computed as the eigenvector of AT  associated to the eigenvalue 1. What is the value of x(k→∞) ?

Note, in MATLAB one computes eigenvalues and eigenvectors with:

[vecs, vals]= eig( A' ); eigenvalues= diag(vals), eigenvectors= vecs

B5. [Tuning the stochastic parameters] Suggest a way of tuning the values α1, α2,  β and γ so that 
xk→∞T=0.1±δ   0.6±δ   0.3±δT where δ<10-4.

Revised by Prof. José Gaspar (2022).

See references

Chapter 7 of "Introduction to dynamic systems; theory, models, and applications", David G. Luenberger, Wiley 1979

Chapter 7 of "Introduction to discrete event systems", C. Cassandras and S. Lafortune, Springer 2008